L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication

A Manuscript Presented to the Faculty of the College of Computing Studies, Information and Communication Technology

Isabela State University – Echague Campus

In Partial Fulfilment of the Requirements for the Degree

Bachelor of Science in Information Technology

By:

Ched Nikko P. Pangilinan

Joshkirby Tyler M. Bote

Naph Yvan P. Villamar

September 2026

TABLE OF CONTENTS

CHAPTER I: INTRODUCTION	4

1.1 Project Context	4

1.1.1 Background of the Study	6

1.1.2 Statement of the Problem	8

1.2 Objectives of the Project	8

1.3 Scope and Limitations	11

1.4 Significance of the Study	12

CHAPTER II: REVIEW OF RELATED LITERATURE AND STUDIES	14

2.1 Introduction	14

2.2 Review of Related Literature	14

2.3 Review of Related Studies	19

2.4 Synthesis of the Reviewed Literature and Studies	20

CHAPTER III: RESEARCH METHODOLOGY	21

3.1 Research Design	21

3.2 System Development Method	23

3.2.1 Requirement Analysis	24

3.2.2 System Design	25

3.2.3 System Development	27

3.2.4 System Testing	27

3.3 Participants/Respondents	30

3.4 Data Gathering Instruments	32

3.5 Evaluation Procedure	37

3.6 Experimental Design	38

3.7 Data Analysis Techniques	40

3.8 System Evaluation Criteria	41

3.9 Ethical Considerations	43

CHAPTER IV: TECHNICAL BACKGROUND	45

4.1 Introduction	45

4.2 System Architecture Overview	46

4.3 IoT Device Architecture	49

4.4 Network Architecture	52

4.5 Application Architecture	54

4.6 Database Design	55

4.7 Data Flow Diagram	58

4.8 Security Architecture	60

4.9 Threat Model and Security Considerations	68

4.10 Security Testing Approach	69

4.11 System Design Considerations	70

4.12 System Limitations	71

4.13 Prototype Implementation and System Interface	72

4.13.1 Hardware Prototype	72

4.13.2 Authentication and Access Control	73

4.13.3 Penetration Testing Results	84

REFERENCES	85

CHAPTER I

INTRODUCTION

1.1 Project Context

Top Achievers Private School Inc. (TAPS) currently uses an RFID-based system installed at the main school entrance. Students tap their RFID cards upon arrival, primarily serving as a notification mechanism for parents to monitor whether their child has entered or left the school premises. However, the system does not record or validate actual classroom attendance.

Based on observation, TAPS accommodates approximately 300–500 students daily, with attendance-related tapping occurring through a single RFID terminal at the gate. During peak hours, an average queue of 20–30 students form at the RFID terminal, resulting in delays and congestion. In some cases, students bypass the RFID terminal to avoid long waiting times, reducing the reliability of attendance monitoring. Inside classrooms, teachers still conduct manual roll calls, consuming approximately 5–10 minutes of the 40-minute class period. The existing system also lacks classroom-level validation and teacher authentication mechanisms.

From a cybersecurity perspective, the current attendance setup lacks sufficient protection mechanisms for attendance data transmitted within the network. The absence of encrypted communication, device authentication, access control, and audit logging increases the risk of unauthorized access, data interception, and possible record tampering, which may compromise the confidentiality, integrity, and reliability of attendance information.

To address these issues, the researchers designed and developed L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication. The developed system implements classroom-based RFID attendance recording combined with biometric teacher verification and multi-layer security mechanisms, improving attendance accuracy, reducing processing time, and strengthening the protection of attendance information within a controlled Local Area Network (LAN) environment. The system captures attendance through RFID readers positioned at classroom entry points rather than at the school gate, and uses fingerprint authentication to verify teacher identity before an attendance session can be opened.

Practical constraints were considered throughout development, including the cost of hardware deployment per classroom, maintenance of devices such as RFID readers and fingerprint sensors, network reliability within the LAN, and deployment feasibility in terms of installation and power requirements.

The system uses IoT-based components suitable for classroom attendance monitoring and local network communication. Fingerprint authentication prevents proxy attendance, which cannot be addressed by RFID alone. A LAN-based architecture was adopted to provide faster communication, reduced latency, and improved security by limiting exposure to external networks.

Through this implementation, the study aims to improve attendance accuracy, reduce processing time, minimize congestion, and enhance the security and reliability of attendance data.

1.1.1 Background of the Study

Educational institutions continue to adopt digital systems to improve operational efficiency; however, existing attendance practices still present limitations in accuracy, validation, and security. At present, the RFID system implemented in schools is primarily used as a notification tool to inform parents whether their son or daughter has entered or left the school premises. It does not record or validate actual classroom attendance, resulting in a gap between entry monitoring and real student presence during instructional time.

Existing attendance systems continue to face challenges related to attendance validation, operational efficiency, and data security. Although RFID-based systems improve entry monitoring, many implementations still lack classroom-level verification, teacher authentication, and sufficient protection mechanisms for attendance information.

In addition, existing systems lack essential security mechanisms such as encryption, access control, and audit logging. This creates vulnerabilities in attendance data, allowing potential unauthorized access or modification, which compromises data integrity and accountability.

To address these issues, this study developed L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication, which enables automated, classroom-level attendance recording. The system uses RFID technology to capture student attendance at the point of instruction and integrates biometric fingerprint authentication to ensure that only authorized teachers can initiate attendance sessions.

The development of the system involved the design and integration of IoT components using the ESP32 microcontroller within a Local Area Network (LAN) to support real-time data transmission.

The system also implements a fallback mechanism in which attendance data is retained on the terminal during connectivity interruptions and synchronized automatically once the network is restored. Each held record carries the timestamp taken at the moment of the tap together with an idempotency key, so that a replayed or retried transmission cannot produce a duplicate attendance record.

To ensure data security and reliability, the system incorporates multi-layer security mechanisms, including encrypted communication for data protection, role-based access control for user authorization, per-device request authentication, and audit logging for traceability of system activities.

Following development, the system will be evaluated for system quality in terms of functionality, usability, and performance, and for security using ISO/IEC 27001, guided by the CIA Triad, in terms of confidentiality, integrity, and availability.

Through this development and evaluation, the study provides a more accurate, efficient, and secure approach to attendance monitoring by addressing the limitations of entry-based systems, reducing time consumption, and ensuring the reliability and integrity of attendance records.

1.1.2 Statement of the Problem

The current attendance monitoring system of Top Achievers Private School Inc. (TAPS) presents several operational and security issues that affect the accuracy, efficiency, and reliability of attendance recording within the school.

Specifically, this study seeks to answer the following questions:

1. How can manual classroom attendance checking, which consumes 5–10 minutes per session, be minimized to improve instructional time?

2. How can the system ensure accurate classroom-level attendance recording?

3. How can congestion at the RFID gate, measured by queue length (20–30 students) and waiting time, be reduced?

4. How can teacher identity verification be integrated to ensure secure and authorized attendance recording?

5. How can multi-layer security mechanisms be implemented to protect attendance data and ensure confidentiality, integrity, and availability?

1.2 Objectives of the Project

General Objective

The main goal of this study is to design, implement, and evaluate L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication that improves classroom-level attendance monitoring, reduces processing time, and ensures data security through the application of robust cybersecurity mechanisms.

Specific Objectives

1. Design and implement an IoT-based hardware system that:

1.1 Captures student attendance via RFID tapping at classroom entry points;

1.2 Verifies teacher identity using biometric fingerprint authentication before activating attendance sessions;

1.3 Automatically records student ID, date, and time for accurate and real-time attendance monitoring.

2. Develop a locally hosted system that:

2.1 Allows administrators to register users and IoT devices;

2.2 Stores attendance records in a centralized database for efficient retrieval;

2.3 Generates attendance reports and dashboards for monitoring and decision-making.

3. Implement selected security mechanisms to protect attendance data and system operations by:

3.1 Securing transmitted data against interception through encrypted communication;

3.2 Preventing unauthorized access through authentication and role-based access control;

3.3 Restricting unauthorized system interaction through controlled network access;

3.4 Improving accountability and monitoring through audit logging of system activities;

3.5 Applying application-level security measures to protect system operations and user information;

3.6 Implementing network security controls such as IP filtering, firewall configuration, encrypted communication, and device authentication to secure IoT-based attendance communication within the Local Area Network (LAN).

4. Evaluate and validate the system by:

4.1 Measuring processing time, queue length, and system efficiency during attendance recording;

4.2 Comparing system-recorded attendance with manual classroom attendance to assess accuracy;

4.3 Assessing system usability, performance, and reliability using measurable indicators, and evaluating system security against ISO/IEC 27001 guided by the CIA Triad.

5. Conduct penetration testing to validate the effectiveness of the implemented security controls by:

5.1 Verifying secure HTTPS/TLS communication between the IoT terminals and the server using Wireshark, confirming that transmitted data is encrypted;

5.2 Testing resistance to brute-force login attempts using Hydra on Kali Linux to confirm that repeated unauthorized login attempts are blocked;

5.3 Performing port scanning using Nmap on Kali Linux to confirm that only authorized ports are exposed on the server;

5.4 Confirming that forged or unregistered device requests are rejected and recorded in the system’s audit and security logs.

1.3 Scope and Limitations

This study focuses on the design, development, and implementation of L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication. The system records student attendance at the classroom level using RFID technology integrated with IoT devices. It includes biometric fingerprint authentication for teacher verification and a locally hosted web-based platform for managing attendance data.

The system implements multi-layer security mechanisms such as encrypted communication, per-device request authentication, Role-Based Access Control (RBAC), network access control, and audit logging. It will be evaluated using ISO/IEC 27001 and the CIA Triad.

The system is limited to classroom-based attendance monitoring within TAPS and does not include gate-level tracking, learning management systems, grading systems, facial recognition, GPS tracking, or mobile application integration. Only teachers and administrators have system access.

The system operates within a Local Area Network (LAN) and is not integrated with cloud-based or external systems. As a result, it has no remote access, limited data sharing, and is not scalable across multiple campuses. In cases of network failure, data is retained on the terminal and transmitted once the connection is restored, which may cause delays in real-time monitoring. The system was developed as a prototype comprising two classroom terminals, so the evaluation covers two classrooms and the results may not transfer uniformly to a full deployment in which every room is instrumented. Performance in large-scale deployment, where increased users and devices may affect efficiency, is therefore outside what this evaluation can establish.

Security testing will be conducted through controlled procedures, including simulated attack scenarios; however, these may not cover all possible real-world attack variations. Evaluation results will be based on a limited number of respondents in a controlled environment, which may affect generalizability.

Additionally, the system requires hardware installation per classroom and ongoing maintenance, which may impact cost and long-term deployment.

1.4 Significance of the Study

School Administrators

Provides accurate and real-time classroom attendance data, improving monitoring and decision-making through secure access and audit logging.

Teachers

Reduces attendance recording time by minimizing manual roll calls and ensures a more efficient and authenticated attendance process.

Students

Improves attendance recording by making it faster, more organized, and less time-consuming during class sessions.

IT Administrators

Enhances system management and strengthens network and data security through the implementation of selected security mechanisms that help protect attendance records and sensitive user information from unauthorized access and modification.

Cybersecurity and Future Researchers

Provides a reference for the development of secured IoT-based systems by demonstrating how selected security mechanisms can be applied to protect sensitive attendance data and improve network security within educational environments.

CHAPTER II

REVIEW OF RELATED LITERATURE AND STUDIES

2.1 Introduction

This chapter presents the review of related literature and studies that provide the technological and security foundations for the development of the proposed Multi-Layer Secured IoT-Based Attendance Monitoring System. The discussion is organized using a thematic approach, grouping related concepts into key areas such as IoT-based attendance systems, identification technologies, and multi-layer security mechanisms. Existing literature shows that automated attendance systems help reduce manual work, speed up attendance recording, and improve data accuracy. However, many systems still lack strong security implementation, especially in terms of multi-layer protection and teacher-level validation. These gaps highlight the need for a more secure, accurate, and integrated attendance monitoring system.

2.2 Review of Related Literature

IoT-Based Attendance Systems

The Internet of Things (IoT) refers to a network of interconnected devices that collect, process, and exchange data through communication networks. In educational systems, IoT is used to automate attendance monitoring by allowing devices to record and send attendance data in real time.

IoT-based attendance systems use hardware components such as RFID readers and microcontrollers to capture attendance data and send it to a centralized server. These systems reduce manual attendance checking, shorten recording time, and allow real-time monitoring of student presence in classrooms (Shrivastava et al., 2023).

However, many IoT-based attendance systems focus mainly on functionality and data transmission. Some systems do not include strong security features such as encryption, device authentication, and monitoring. This makes them vulnerable to unauthorized access, data interception, and system misuse. Recent studies explain that IoT systems are exposed to security risks because devices constantly communicate and exchange data over networks (Ferrag et al., 2021; Neshenko et al., 2021).

Identification Technologies in Attendance Systems

Attendance monitoring systems commonly use RFID and biometric authentication technologies to improve attendance accuracy and reduce manual recording. RFID enables fast identification through contactless card scanning, while biometric authentication provides stronger identity verification using unique physiological characteristics such as fingerprints (Shrivastava et al., 2023).

RFID Technology for Student Attendance

RFID (Radio Frequency Identification) is commonly used in attendance systems because it is fast and easy to use. Students are given RFID cards, and attendance is recorded when the card is tapped on a reader. The system records the student's identity along with the date and time.

RFID systems reduce the time needed to record attendance and remove the need for manual roll calls. This allows faster and more efficient attendance tracking inside the classroom.

Although RFID technology improves attendance efficiency, it has limitations in identity verification because RFID cards may be shared or used by unauthorized individuals. This creates risks of proxy attendance and inaccurate attendance records in educational environments (Shrivastava et al., 2023).

Fingerprint-Based Teacher Authentication

Fingerprint authentication uses the unique pattern of a person's fingerprint to verify identity. In attendance systems, it ensures that only authorized individuals can access or activate the system.

Biometric authentication strengthens attendance reliability by ensuring that only registered individuals can access or activate the attendance system. Fingerprint authentication is widely recognized as a secure authentication mechanism because fingerprint patterns are unique and difficult to duplicate (Ahmad et al., 2022).

However, most systems use fingerprint authentication for student attendance only. There are few systems that use fingerprint verification to confirm the presence of the teacher before attendance recording. Without teacher validation, attendance can be recorded even if the teacher is not present.

Applying fingerprint authentication for teacher-level verification improves accountability and ensures that attendance sessions are initiated only by authorized personnel. This additional authentication layer enhances attendance validity and reduces opportunities for unauthorized attendance recording (Ahmad et al., 2022).

Security in IoT-Based Systems

Security is an important part of IoT-based systems because data is sent through networks where it can be accessed or attacked. IoT systems are exposed to risks such as unauthorized access, data interception, and device misuse because devices continuously communicate with each other (Ferrag et al., 2021; Neshenko et al., 2021).

Common security problems include lack of encryption, weak authentication, and no monitoring system. These issues can allow attackers to access, modify, or misuse attendance data. To prevent these risks, modern systems use security mechanisms such as encryption, authentication, access control, and monitoring. These measures help protect data and ensure that the system works correctly and securely (Ferrag et al., 2021).

Multi-Layer Security Frameworks

Multi-layer security, also known as defense-in-depth, uses multiple security mechanisms across different parts of a system. This approach ensures that if one layer fails, other layers still protect the system.

In attendance systems, multi-layer security includes encrypting data during transmission, verifying users and devices, controlling access through roles, and recording system activities through audit logs.

These layers work together to protect attendance data and system operations. Using multiple security layers reduces the chance of system compromise by addressing different types of security risks at different levels (Ferrag et al., 2021).

Network Security in IoT Systems

Network security protects communication between devices and controls which devices are allowed to access the system. In IoT systems, devices constantly exchange data, making them vulnerable if network protection is weak.

Security controls such as IP filtering, firewall configuration, and port restriction are used to limit access to authorized devices only. These controls are important in Local Area Network (LAN) environments.

These mechanisms help prevent unauthorized access, packet interception, and network intrusion in IoT systems (Neshenko et al., 2021). Without network security, attackers may access the system and cause data breaches or disruptions.

A LAN-based deployment also reduces the attack surface of the system by limiting exposure to external internet-based threats compared to cloud-dependent architectures. This approach strengthens internal network control, improves communication speed, and enhances monitoring of connected IoT devices.

Application Security and Data Protection

Application security protects the system from internal threats by ensuring that only valid inputs and authorized users can access system functions. This includes input validation, password hashing, and secure session management. These controls prevent invalid data and unauthorized access.

Audit logging is also important because it records system activities such as login attempts and attendance transactions. These logs help administrators track system usage, detect unusual behaviour, and ensure accountability. These mechanisms ensure that attendance data is not changed, accessed by unauthorized users, or lost during system operation (Ahmad et al., 2022; Whitman & Mattord, 2021).

Security Standards and Evaluation Models

Standard frameworks are used to ensure that the system is reliable and secure. The CIA framework focuses on confidentiality, integrity, and availability, ensuring that data is protected, accurate, and accessible (Whitman & Mattord, 2021).

ISO/IEC 27001 provides guidelines for implementing security controls such as encryption, authentication, access control, and monitoring (International Organization for Standardization, 2022). These frameworks help ensure that the system is secure and performs properly.

2.3 Review of Related Studies

Foreign Related Studies

Studies such as those by Shrivastava et al. (2023) show that IoT-based attendance systems reduce manual work, improve accuracy, and allow real-time monitoring of attendance data.

However, many studies focus mainly on system functionality and do not include strong security features such as encryption, authentication, and monitoring. Some systems rely only on RFID, which makes them vulnerable to proxy attendance. Other systems use biometric authentication but focus only on student verification and do not confirm teacher presence. In addition, many systems do not implement multi-layer security, especially in network protection and audit logging.

Local Related Studies

Recent local and international implementations of digital attendance systems demonstrate improvements in attendance recording efficiency, data accessibility, and monitoring capabilities. However, many systems still focus mainly on automation and functionality while providing limited implementation of encryption, access control, audit logging, and network-level security protections.

2.4 Synthesis of the Reviewed Literature and Studies

The reviewed literature shows that modern attendance systems using IoT, RFID, and fingerprint technologies improve attendance recording by making it faster, more accurate, and easier to manage.

However, several gaps still exist. Many systems focus only on functionality and do not include strong security features. Most systems lack multi-layer security, including encryption, device authentication, network protection, and audit logging.

In addition, many systems use only one identification method. RFID systems can be misused through proxy attendance, while biometric systems are often limited to student verification only. Another major issue is the lack of teacher verification before attendance recording. Most systems do not ensure that attendance is recorded only when the teacher is present.

To address these gaps, this study proposes L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication that combines RFID-based student attendance with fingerprint-based teacher authentication. The system applies multi-layer security, including encryption, authentication, access control, network protection, audit logging, and application security.

This approach improves attendance accuracy, ensures teacher validation, and provides better protection of attendance data.

CHAPTER III

RESEARCH METHODOLOGY

3.1 Research Design

This study employed a multi-method research design, integrating Design and Development Research, Experimental Research, and Survey Research to ensure a comprehensive and systematic approach in developing and evaluating L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication.

Design and Development Research served as the primary framework of the study, guiding the systematic process of designing, developing, and testing the system based on the problems identified in Chapter I, including manual attendance checking, lack of classroom-level validation, absence of teacher authentication, congestion during attendance, and weak security mechanisms. This phase has been completed, and its output is the working prototype documented in Chapter IV.

Experimental Research will be used as a supporting method to compare the existing manual attendance system and the developed system using measurable variables such as processing time, waiting time, queue length, accuracy, efficiency, response time, and security effectiveness. These variables are directly derived from the study objectives and will allow quantitative evaluation of system performance.

Survey Research will be applied to gather user feedback using a structured questionnaire whose items are drawn from key system-quality characteristics, allowing measurement of perceived system quality on a five-point Likert scale.

Evaluation will be embedded within the study to assess perceived system quality, and to assess system security using ISO/IEC 27001, guided by the CIA framework (Confidentiality, Integrity, and Availability).

The methodological flow of the study follows this sequence: system design and development, system testing, experimental comparison, survey-based evaluation, and data analysis. As of this prototype presentation, system design and development have been completed and development-time testing has been performed. The experimental comparison, survey-based evaluation, and data analysis will be conducted upon deployment of the prototype at the partner school.

The study involved the development and evaluation of three main components: IoT-based hardware for attendance capture, software systems for data processing and management, and security mechanisms for data protection. These components were integrated and will be evaluated collectively. The methodological flow of the study is presented in Figure 1.

Figure 1

Methodological Flow of the Study

3.2 System Development Method

The study followed an Iterative Software Development Life Cycle (SDLC) model, in which the system was developed through repeated cycles of planning, design, development, testing, and refinement. This approach allowed continuous improvement based on testing results and user feedback. Security mechanisms were implemented consistently across all phases of development to ensure data protection and system integrity. The development cycle followed is presented in Figure 2.

Figure 2

Iterative System Development Life Cycle Diagram

3.2.1 Requirement Analysis

In this phase, the researchers analyzed the existing attendance monitoring system and identified operational and security issues, including time-consuming manual attendance checking, lack of classroom-level validation, absence of teacher identity verification, congestion during attendance recording, and vulnerabilities in data security. To address these issues, the system introduced RFID-based attendance recording at classroom entry points and biometric fingerprint authentication for teacher verification, ensuring accurate attendance recording and authorized system usage. Multi-layer security mechanisms such as encrypted communication, authentication, access control, and audit logging were integrated to protect system data.

3.2.2 System Design

The system was designed to operate within a secure Local Area Network (LAN) to ensure fast, reliable, and low-latency communication. The design includes a database structure for managing users, students, attendance records, and logs; a web-based dashboard for monitoring and administration; and a controlled workflow in which attendance recording occurs only after successful teacher authentication.

The network security design includes network-level access control by address range, controlled port access, firewall configuration, per-device request authentication using signed requests, and encrypted communication using HTTPS/TLS to minimize unauthorized access and reduce network exposure. The overall system design is presented in Figure 3.

Figure 3

System Design Diagram

3.2.3 System Development

The system was developed using IoT-based hardware and software components, including the MFRC522 RFID reader module, the AS608 optical fingerprint module, and the ESP32 microcontroller, integrated with a web-based platform for monitoring and data management. The web application was implemented in PHP and backed by a MariaDB relational database, and is designed to run on a single server within the school’s Local Area Network. Security features including encrypted communication, authentication, access control, and audit logging were implemented to ensure confidentiality, integrity, and availability of data.

User passwords are secured using bcrypt hashing at cost factor 12, and a distinct cryptographic credential is generated for each registered IoT device to authenticate and sign its communication with the server.

3.2.4 System Testing

Structured testing was carried out in two stages. Development-time verification was performed alongside implementation to confirm the correctness and security of the system as it was built. Evaluation-stage testing, described in the remainder of this chapter, will be conducted upon deployment of the prototype at the partner school.

Two automated test suites were developed and executed during implementation. A certificate and transport-security suite verifies the system’s TLS configuration, and returned 40 passing assertions covering encryption of the root key at rest, trust-chain validation, the certificate extensions that browsers and embedded TLS clients actually check, preservation of the root certificate across reissue so that deployed terminals continue to validate, and refusal of malformed hostname input. A concurrency and correctness suite exercises the live service layer against a real database to verify row locking, unique constraints, and deadlock retry behaviour when simultaneous taps are recorded. The output of the transport-security suite is presented in Chapter IV as evidence of the implemented transport protection.

Functional testing will verify that all system components operate correctly, including attendance recording and authentication processes, with a success rate of at least 95 percent indicating acceptable functionality.

Performance testing will measure system response time and latency, with the system expected to respond within less than two seconds to support real-time operation.

All penetration testing activities will be limited strictly to owned and authorized LAN devices within a controlled academic environment to simulate common security threats and evaluate the effectiveness of implemented protection mechanisms without affecting external or real-world systems.

Penetration testing using Wireshark and the Kali Linux toolset (Hydra and Nmap) will be limited strictly to authorized and owned Local Area Network (LAN) devices for academic and evaluation purposes only. Wireshark will be used for packet inspection and verification of encrypted traffic to analyze communication between IoT devices and the server. Nmap will be used for port and service discovery to identify unnecessary open ports and evaluate network exposure within the LAN environment. Hydra will be used to conduct controlled brute-force login attempts to assess the effectiveness of the account lockout and rate-limiting mechanisms against unauthorized access. No external systems or unauthorized networks will be targeted during testing procedures.

Table 1

Security Plan Test

| Threat | Tool | Test Procedure | Expected Result | Metric | Pass/Fail Basis |
| --- | --- | --- | --- | --- | --- |
| Unauthorized Access | Hydra (Kali Linux) | Conduct brute-force login attempts against the authentication endpoint | Access blocked after lockout threshold | Number of successful unauthorized attempts | No successful unauthorized access |
| Packet Interception | Wireshark | Monitor transmitted packets | Encrypted packets observed | Encrypted vs unencrypted packets | No readable sensitive data |
| Open Port Exposure | Nmap | Scan server ports | Only authorized ports open | Number of open ports | No unnecessary open ports |
| Device Spoofing | Kali Linux (crafted HTTP request) | Attempt access with an unregistered device or missing request signature | Device rejected and event logged | Rejected unauthorized requests | 100% rejection |
| Log Monitoring | Audit Logs | Perform login and attendance actions | Logs generated correctly | Audit log completeness | Complete logs generated |

These tools were selected due to their effectiveness in analyzing network traffic, identifying vulnerabilities, and evaluating system security within a controlled academic environment. Usability testing will be conducted through actual user interaction using survey instruments. All testing will be conducted in a controlled environment without affecting real-world systems.

3.3 Participants/Respondents

The scope of the evaluation is bounded by the hardware constructed for the       prototype. Two classroom terminals were built, so two classrooms can be instrumented at any one time, and the study will be conducted in two classrooms at Top Achievers Private School Inc. The two rooms were selected through purposive sampling, on the basis of scheduling availability and the practicality of installing and powering a terminal in each room. Within the two rooms the study will use total enumeration, also called complete enumeration or a census, in which every member of the accessible population is included rather than a subset drawn from it. The study therefore combines two techniques: purposive sampling determines the study site, and total enumeration determines the respondents.

The accessible population and its role in the evaluation are shown in Table 2. A total of 52 respondents will take part.

Table 2

Respondents and Their Role in the Evaluation

| Respondent group | Number | Role in the evaluation |
| --- | --- | --- |
| Students (two classrooms) | 42 | Attendance capture during actual class sessions; source of processing time, waiting time and queue length measurements; respondents to the short form of the perceived-quality questionnaire |
| Teachers | 8 | Session initiation through fingerprint verification; respondents for perceived system quality |
| School administrator | 1 | Monitoring capability, data reliability and decision-making support |
| IT administrator | 1 | System performance, network reliability and effectiveness of the implemented security mechanisms |
| Total | 52 |  |

Total enumeration is appropriate for this study for three reasons. First, the accessible population is small enough to be included in full, so nothing is gained by drawing a sample from it. Second, attendance is a whole-class process: measuring waiting time and queue length requires every student present to pass through the terminal, and excluding any student would systematically understate the congestion that the study exists to measure. Third, the teacher, administrator and IT administrator groups are already at their full size within the study site, and there is no meaningful way to draw a sample from a single IT administrator.

Since every member of the accessible population is included, the study reports findings that describe this site, as stated in the Scope and Limitations in Chapter I.

The student respondents are minors, informed consent will be obtained from a parent or guardian in addition to the assent of the student, as described in the Ethical Considerations in Section 3.9.

3.4 Data Gathering Instruments

The study will use multiple instruments to collect quantitative data for system evaluation. The selected instruments are directly aligned with the study objectives to ensure that each objective is measured using appropriate and measurable variables. System-generated data such as attendance logs, timestamps, response time records, and database entries will be used as primary instruments to measure processing time, response time, efficiency, and accuracy. Observation tools such as checklists and time recording devices will be used to measure queue length and waiting time during attendance procedures. Record comparison between manual attendance and system-generated attendance will be used to determine accuracy.

The instruments used in the study are categorized based on system components. For the IoT component, RFID logs, device timestamps, and response time measurements will be used to evaluate device performance and communication efficiency. For the software component, database records, system outputs, and user interaction data will be used to evaluate functionality, usability, and reliability. For the security component, tools such as Wireshark and Nmap will be used to identify vulnerabilities and evaluate the effectiveness of implemented security mechanisms. The survey instrument will be a five-point Likert scale questionnaire whose items address key system-quality characteristics. Every instrument in the study yields numeric data.

Table 3

Likert Scale Interpretation

| Scale | Description | Interpretation |
| --- | --- | --- |
| 5 | Strongly Agree | Very High |
| 4 | Agree | High |
| 3 | Neutral | Moderate |
| 2 | Disagree | Low |
| 1 | Strongly Disagree | Very Low |

A researcher-made questionnaire will be used to measure perceived system quality. Its items address key system-quality characteristics: functional suitability, performance efficiency, usability, and reliability. They are answered on the five-point Likert scale interpreted in Table 3. The instrument is administered in two forms. Teachers, the school administrator and the IT administrator answer the full form, which covers every characteristic, since they interact with session control, monitoring, reporting and administration. Students answer a short form limited to the part of the system they actually use, covering the ease and speed of tapping and the clarity of the terminal’s response. Administering the short form to all 42 students rather than to the teaching staff alone means the grand mean rests on the whole accessible population, which makes it far less sensitive to a small number of divergent ratings. System security will be evaluated against ISO/IEC 27001, guided by the CIA framework of confidentiality, integrity and availability, using the results of security testing and the completeness of the recorded audit and security logs as evidence.

Prior to actual deployment, the survey questionnaire will undergo pilot testing to evaluate clarity, consistency, and reliability. Selected respondents outside the target participants will be asked to complete the questionnaire to identify unclear statements and improve the validity of the instrument.

Each objective of the study will be evaluated using corresponding metrics and instruments. The objective of reducing attendance time will be measured using processing time and waiting time obtained from system logs and observation tools. The objective of reducing congestion will be evaluated using queue length measurements. The objective of improving attendance accuracy will be measured through record comparison between manual and system-generated data. The objective of improving system efficiency will be evaluated through computed efficiency percentages. The objective of ensuring system security will be assessed through security testing tools by identifying vulnerability severity levels.

Teacher authentication effectiveness will be measured based on the successful authentication rate and the number of failed authentication attempts recorded during system usage. This will ensure that only authorized teachers are able to initiate attendance sessions.

Security effectiveness will be evaluated based on successful prevention of unauthorized access attempts, protection of transmitted attendance data, enforcement of authentication procedures, and completeness of recorded audit logs during controlled testing.

System reliability will also be evaluated by testing the fallback mechanism, in which attendance data is temporarily retained on the terminal during network interruptions and synchronized once the connection is restored.

Table 4

Instrument–Metric Mapping

| Objective | Metric | Instrument |
| --- | --- | --- |
| Reduce attendance time | Processing Time | System logs, stopwatch |
| Reduce congestion | Waiting Time | Observation, stopwatch |
| Reduce congestion | Queue Length | Observation checklist |
| Improve accuracy | Accuracy Rate | Record comparison |
| Improve efficiency | Efficiency (%) | Computation |
| Ensure security | Vulnerability severity | Wireshark, Nmap |
| Verify teacher identity | Authentication success rate | System logs |
| Ensure reliability | Data synchronization success | System logs |

Table 5

SOP–Objectives–Metrics–Instrument Mapping

| Statement of the Problem | Objective | Metric | Instrument | Success Criterion |
| --- | --- | --- | --- | --- |
| How can manual classroom attendance checking be minimized? | Reduce attendance recording time | Processing Time | System logs, stopwatch | Reduced processing time |
| How can accurate classroom level attendance recording be ensured? | Improve attendance accuracy | Accuracy Rate | Record comparison | ≥ 95% |
| How can congestion at the RFID gate be reduced? | Reduce waiting time and queue length | Waiting Time, Queue Length | Observation tools | Reduced congestion |
| How can teacher identity verification be integrated? | Verify authorized teacher access | Authentication Success Rate | System logs | ≥ 95% |
| How can multi-layer security mechanisms protect attendance data? | Implement network and application security | Vulnerability Severity, Unauthorized Access Attempts | Wireshark, Nmap, Hydra | No High or Critical vulnerabilities |

3.5 Evaluation Procedure

The evaluation process will follow a structured sequence: system deployment, functional and performance testing, user testing in actual attendance scenarios, experimental comparison between the manual and the developed system, survey distribution to respondents, and data collection and analysis.

Each instrument will follow a specific evaluation procedure. System logs will be collected during attendance sessions to measure processing time and response time. Observation tools will be used during actual attendance scenarios to record waiting time and queue length. The system-quality questionnaire will be administered after system use to measure perceived system quality. Security testing tools will be applied in controlled scenarios to identify vulnerabilities and assess system protection mechanisms. The collected data will then be analyzed based on predefined metrics and evaluation criteria.

Controlled simulation scenarios will be conducted to generate measurable evidence supporting system performance and security claims. These simulations will include attendance scenarios for performance testing, authentication testing for teacher verification, controlled attack scenarios for security evaluation, and network interruption scenarios to validate the fallback mechanism.

The evaluation of the system will be conducted across its major components, including the IoT layer (device performance), network layer (data transmission efficiency), application layer (system functionality and usability), and security layer (data protection and access control).

3.6 Experimental Design

The study will compare the existing manual attendance system and the developed system using measurable and quantitative criteria derived from the study objectives. Processing time will measure the reduction in attendance duration, waiting time and queue length will measure congestion levels, accuracy will evaluate the correctness of attendance records, efficiency will determine the improvement between manual and automated processes, response time will assess system performance, and security effectiveness will evaluate the strength of implemented security mechanisms.

Processing time will be computed as the total time required to complete attendance divided by the number of students, while waiting time will be calculated as the difference between arrival time and completion time. Accuracy will be measured as the percentage of correct records, and efficiency will be computed as the percentage improvement between manual and system performance.

In addition, authentication performance will be evaluated based on the rate of successful and failed fingerprint authentication attempts, while system reliability will be measured through the successful synchronization of retained data after simulated network interruptions. Security effectiveness will be analyzed based on the system’s ability to prevent unauthorized access, detect abnormal activities, and maintain data integrity during controlled testing scenarios.

All experiments will be conducted under the same class size, environment, time duration, and attendance procedures to ensure a fair and valid comparison.

The experimental process used to compare the manual attendance system and the developed system is illustrated in Figure 4. The diagram presents the sequence of steps including setup, execution, data collection, and result comparison.

Figure 4

Experimental Setup and Process Flow Diagram

Null Hypothesis (H₀):

There is no significant difference between the manual attendance system and the developed system in terms of processing time, accuracy, and efficiency.

Alternative Hypothesis (H₁):

There is a significant difference between the manual attendance system and the developed system in terms of processing time, accuracy, and efficiency.

Statistical Test

To determine whether the observed differences between the manual attendance system and the developed system are statistically significant, the study will use a paired sample t-test. This test is appropriate because the same group of participants will be evaluated under two conditions: manual attendance and system-based attendance.

The paired t-test will analyze differences in processing time, accuracy, and efficiency for each participant or attendance session. A significance level of 0.05 will be used. If the computed p-value is less than 0.05, the null hypothesis will be rejected, indicating that the developed system provides a statistically significant improvement over the manual attendance system.

Mathematical Models

The study will use the following formulas to measure system performance:

Accuracy Rate: Accuracy (%) = (Number of Correct Attendance Records / Total Attendance Records) × 100

Processing Time: Processing Time = Total Time to Record Attendance / Number of Students

Efficiency: Efficiency (%) = ((Manual Attendance Time − System Attendance Time) / Manual Attendance Time) × 100

Waiting Time: Waiting Time = Time of Completion − Time of Arrival

3.7 Data Analysis Techniques

Data collected in the study will be analyzed using descriptive and comparative statistical methods. Mean, median, and standard deviation will be used to analyze survey responses and evaluate user perception, while comparative analysis will be used to determine differences between manual and system performance.

The results of the analysis will be interpreted based on predefined success criteria. For survey data, weighted mean and grand mean will be used to determine user perception levels. For experimental data, comparative analysis will be used to determine improvements between the manual and the developed system. The system will be considered acceptable if it meets the defined thresholds for usability, accuracy, performance, and security.

Table 6

Interpretation of Mean Scores

| Mean Range | Interpretation |
| --- | --- |
| 4.20–5.00 | Very High |
| 3.40–4.19 | High |
| 2.60–3.39 | Moderate |
| 1.80–2.59 | Low |
| 1.00–1.79 | Very Low |

3.8 System Evaluation Criteria

Table 7

System Evaluation Criteria

| Criteria | Indicator | Measurement | Success Criteria |
| --- | --- | --- | --- |
| Perceived system quality | System-quality Likert questionnaire | Questionnaire | Mean ≥ 3.40 (High) |
| Performance | Response Time | System logs | < 2 seconds |
| Accuracy | Attendance Records | Record comparison | ≥ 95% |
| Authentication | Success Rate | System logs | ≥ 95% |
| Reliability | Data Sync Success | System logs | ≥ 95% |
| Security | Vulnerability Severity | Security testing | No high-risk |
| Network Security | Open Ports | Nmap Scan | No unnecessary open ports |
| Access Control | Failed Unauthorized Requests | Security Testing | 100% rejection of unauthorized requests |
| Access Control | Encrypted Packets | Wireshark Analysis | No readable sensitive data |
| Monitoring | Audit Log Completeness | Audit Logs | Complete logs generated |

The security criteria in Table 7 are organised under the CIA framework, following the guidance of ISO/IEC 27001. Confidentiality is assessed through the absence of readable sensitive data in captured traffic and the rejection of unauthorised requests. Integrity is assessed through the accuracy of attendance records, the completeness of the audit trail, and the system’s refusal of tampered or replayed requests. Availability is assessed through response time and the successful synchronisation of attendance data retained during network interruption. The study applies ISO/IEC 27001 as a guiding framework at the level of confidentiality, integrity and availability.

The success criterion for perceived system quality is a grand mean of 3.40 or above, which Table 6 interprets as High. The obtained mean will be reported together with its verbal interpretation, so that a result in the Very High band is recorded as such even though the acceptance criterion sits at High. The criterion is deliberately set above the midpoint of the scale: a threshold that a merely Moderate rating could satisfy would not distinguish an acceptable system from an indifferent one.

Security performance will be evaluated based on the effectiveness of implemented protection mechanisms during controlled testing. High-risk vulnerabilities must not be present, medium-risk vulnerabilities must be minimized, and low-risk vulnerabilities will be acceptable within controlled limits.

The selected evaluation thresholds are based on acceptable standards commonly used in system performance, usability, and security evaluation studies. High accuracy, fast response time, and reliable authentication are necessary to ensure that the system can operate effectively within classroom environments.

3.9 Ethical Considerations

The study will comply with Republic Act No. 10173, or the Data Privacy Act of 2012. Informed consent will be obtained from all participants prior to data collection, and participation will be voluntary. All collected data will be treated with strict confidentiality and will be used solely for academic purposes. Sensitive information, including biometric and attendance data, will be securely stored and will only be accessed by authorized personnel.

In the developed system, fingerprint data is not stored as an image. The fingerprint sensor converts each enrolled fingerprint into a numeric template held in the sensor module, and the server stores only a reference to that template together with the identity it belongs to. This design limits the personal data held by the system and supports the confidentiality requirement of the Data Privacy Act.

CHAPTER IV

TECHNICAL BACKGROUND

4.1 Introduction

This chapter presents the technical design, system architecture, and implementation of L-SIAMS: A Local IoT-Based Multi-Layer Secured Attendance Monitoring System with RFID and Biometric Authentication for Top Achievers Private School Inc. (TAPS). The system integrates IoT-based hardware, a web-based application, and multi-layer security mechanisms to provide accurate, efficient, and secure attendance monitoring.

The system consists of three major components: the IoT device for attendance capture, the application system for data processing and management, and the security architecture for data protection. These components operate within a Local Area Network (LAN) to provide reliable communication, low latency, and reduced exposure to external threats. The movement of data through the layers of the system is presented in Figure 5.

Figure 5

Data Flow Across System Layers

4.2 System Architecture Overview

The system adopts a multi-layer architecture composed of six main layers: the Device Layer, Network Layer, Transport Layer, Application Layer, Database Layer, and Monitoring Layer. Each layer performs a specific role while interacting with other layers to ensure overall system functionality.

The Device Layer captures attendance data using RFID readers and fingerprint sensors connected to the ESP32 microcontroller. The Network Layer manages secure communication between IoT devices and the server through the Local Area Network. The Transport Layer secures transmitted data using HTTPS/TLS encryption. The Application Layer processes attendance records, authentication requests, and system operations. The Database Layer manages secure storage of attendance records, user credentials, and audit logs. The Monitoring Layer handles activity logging, anomaly detection, and audit trail generation for accountability and security monitoring.

The architecture directly addresses the problems identified in Chapter I. Classroom-based RFID attendance reduces congestion at entry points, while automated recording minimizes manual attendance time. Biometric authentication provides teacher verification, and the integration of multi-layer security mechanisms protects attendance data from unauthorized access and modification.

The overall structure of the system is illustrated in Figure 6, showing the interaction between IoT devices, network components, application systems, and security layers.

Figure 6

System Architecture Diagram

4.3 IoT Device Architecture

The IoT device is built using the ESP32 microcontroller, which serves as the central processing unit responsible for handling input, processing data, and transmitting information to the server. The ESP32 was selected for its cost efficiency, built-in Wi-Fi capability, and suitability for IoT applications.

The device integrates RFID technology to identify students and a biometric fingerprint sensor to verify teacher identity. RFID provides fast and contactless student identification, while fingerprint authentication prevents proxy attendance and ensures that only authorized teachers can initiate attendance sessions.

The system uses the MFRC522 RFID module for student identification due to its low cost, ease of integration, and compatibility with the ESP32 microcontroller. For biometric authentication, the AS608 optical fingerprint module is used for its reliability, built-in template storage, and suitability for embedded systems. These components were selected to ensure affordability, efficiency, and ease of deployment in classroom environments.

Since a fingerprint reader can fail to read a legitimate teacher, whether through a wet, dry or injured finger or a sensor fault, the system provides a controlled failover in which an assigned teacher may open a session using their own account password instead. The failover is deliberately not silent: it is recorded as a distinct security event so that an administrator can see how often biometric verification is being bypassed and by whom.

The IoT device captures student and teacher data, processes the information, and transmits it to the server in a structured format through the network. The structure of the device is presented in Figure 7.

Figure 7

IoT Device Diagram

4.4 Network Architecture

The system operates within a Local Area Network (LAN) using a client-server model. The LAN-based setup was selected to provide faster communication, reduced latency, and enhanced security by limiting exposure to external networks.

Compared to cloud-based deployment, the LAN architecture reduces the system’s attack surface by restricting external internet exposure and allowing stricter internal network monitoring and access control.

The network connects IoT devices, the server, and client devices such as administrator and teacher computers. The Wi-Fi router acts as the central communication hub, enabling data transmission between devices.

Attendance data collected from IoT devices is transmitted to the server, where it is processed and stored. Client devices access the system through a web interface to view attendance records and generate reports.

Network access control is enforced at the application boundary in addition to the router. Every incoming request is checked against configured address ranges before any authentication is attempted: one range for browser access by staff, and a separate range for terminal traffic. A request originating outside its permitted range is refused and recorded as a security event, regardless of whether it carries valid credentials. This makes network position an independent condition of access rather than a consequence of the firewall alone. The network architecture is presented in Figure 8.

Figure 8

Network Architecture

4.5 Application Architecture

The system follows a three-tier architecture consisting of the presentation layer, application layer, and data layer. The presentation layer serves as the user interface, allowing administrators and teachers to interact with the system. The application layer processes requests, applies system logic, and enforces rules such as authentication and validation. The data layer stores system data, including user information, attendance records, and audit logs.

The backend is implemented in PHP with a MariaDB relational database, selected for their compatibility, lightweight footprint, and suitability for LAN-based deployment on modest server hardware. The application runs without third-party runtime dependencies: the class autoloader, the spreadsheet and PDF report writers, the WebSocket server, the chart renderer, and the icon set are all implemented within the project. This decision follows directly from the deployment constraint, since the target network may have no route to the internet from which packages could be installed or updated.

Requests that change attendance state carry an idempotency key. If a terminal retries a transmission because it did not receive a response, the server recognizes the repeated key and returns the original outcome instead of recording a second attendance entry. This protects record integrity across an unreliable link without requiring the terminal to reason about what the server already received.

Monitoring screens receive updates over a WebSocket connection, with Server-Sent Events and long polling as automatic fallbacks when a client or network path does not support the primary transport. Each event carries a sequence number, so a client that reconnects after an interruption requests only the events it missed rather than losing them or reloading the entire view.

The system generates measurable data such as timestamps, response time, authentication logs, and attendance records, which will be used in the experimental evaluation described in Chapter III. These data outputs support the measurement of system performance, accuracy, and efficiency.

4.6 Database Design

The system database uses a relational structure to ensure efficient data storage, retrieval, and data integrity. It is implemented in MariaDB and consists of multiple interrelated tables that support attendance recording, user authentication, and system monitoring.

The main entities include the user table, which stores login credentials and user roles; the teacher table, which is linked to user accounts and stores biometric references; the class table, which organizes students and assigns teachers; and the student table, which contains student identity information along with RFID unique identifiers.

The attendance table records all attendance transactions, while the device table stores information about registered IoT devices and their corresponding credentials. The audit log table records system activities for monitoring and accountability purposes, and a separate security log table records security-relevant events for investigation.

The relationships between these entities are structured to ensure proper data linkage and consistency. A one-to-one relationship exists between the user and teacher entities, while a teacher can handle multiple classes. Each class contains multiple students, and each student can have multiple attendance records. Similarly, a device can send multiple attendance records, and a user can generate multiple audit logs. This relational design ensures traceability, consistency, and secure management of attendance data throughout the system. The relationships between these entities are presented in Figure 9.

Figure 9

Entity Relationship Diagram

4.7 Data Flow Diagram

The Data Flow Diagram illustrates the internal processes of the system, including attendance capture, data validation, and storage. The process begins with teacher authentication, followed by student RFID scanning. The system validates the data and transmits it to the server, where it is processed and stored in the attendance database. The system also generates audit logs for monitoring and traceability, as shown in Figure 10.

Figure 10

Data Flow Diagram

4.8 Security Architecture

The system implements a multi-layer security architecture to ensure data protection and system integrity. The security model comprises device authentication, network security, transport security, application security, database security, and monitoring mechanisms, as shown in Figure 11. Each layer is designed so that the failure of any single control does not by itself grant access to attendance data.

Device Layer Security

Each registered terminal holds two separate credentials: an API key that identifies it, and a 256-bit secret used to sign its requests. Identification and proof of identity are therefore distinct, so a key observed in transit or read from a device cannot by itself be used to impersonate that terminal. Every request from a terminal carries a signature computed over the request content using the device secret, together with a timestamp and a single-use value called a nonce.

The server validates every device request through a fixed six-step sequence, shown in Table 8. Each step has its own failure code, and every failure is written to the security log. The codes returned to the caller are deliberately coarse, so that a rejected request does not reveal which condition failed and therefore does not help an attacker map the system’s defences, while the log retains the precise reason for the administrator.

Table 8

Device Request Validation Sequence

| Step | Condition checked | Failure code | Security purpose |
| --- | --- | --- | --- |
| 1 | The device is registered and active | DEVICE_UNKNOWN | Rejects traffic from any terminal the school has not provisioned |
| 2 | The API key is valid and unexpired | API_KEY_INVALID | Rejects revoked, expired, or fabricated credentials |
| 3 | The request signature is valid | SIGNATURE_INVALID | Proves possession of the device secret; defeats key-only spoofing and detects tampering with the request body |
| 4 | The timestamp is within ±30 seconds | TIMESTAMP_EXPIRED | Bounds the window in which any captured request could be reused |
| 5 | The nonce has not been used before | REPLAY_DETECTED | Prevents a captured valid request from being replayed even inside the time window |
| 6 | The device is bound to the classroom | DEVICE_CLASSROOM_MISMATCH | Prevents a terminal removed from one room from recording attendance for another |

Credentials are managed over their whole life rather than issued once. Keys are generated during device registration, stored in protected form, validated on every request, and may be rotated, with a grace period during which the previous key remains valid so that rotation does not interrupt attendance capture. The system also revokes a device credential automatically when its behaviour indicates compromise: repeated signature failures, or use of the same key from several distinct network addresses within a short period.

Network Layer Security

Network security is enforced through controlled LAN access, including address-range filtering applied to every request before authentication, firewall configuration, and port restriction. Browser traffic and terminal traffic are governed by separate permitted ranges, so a compromised staff workstation does not inherit the network position of a classroom terminal. Requests arriving from outside the permitted range are refused and recorded, whether or not they carry valid credentials.

Transport Layer Security

HTTPS/TLS secures communication between the ESP32 terminals and the server within the LAN. Because the system is designed to run without internet access, it cannot obtain certificates from a public certificate authority. The system therefore operates its own internal certificate authority: it issues the server certificate, keeps the authority’s private key encrypted at rest, and provides the root certificate in a form the firmware compiles in, so that each terminal validates the server it connects to rather than trusting any host that answers. Reissuing the server certificate deliberately preserves the existing root, so that terminals already issued with the root certificate continue to validate without being reflashed.

The correctness of this arrangement was verified by an automated test suite during development. Its results are presented in Section 4.13 as Figure 18.

Application Layer Security

Application-level security combines several controls. User passwords are hashed with bcrypt at cost factor 12, a deliberately slow function chosen because human-chosen passwords are low in entropy and must be expensive to test. Password policy requires a minimum length of twelve characters with mixed character classes, rejects passwords appearing in a common-password blocklist, and prevents reuse of recent passwords. Repeated failed sign-in attempts lock an account for a fixed period.

Sessions are identified by cryptographically random tokens rather than predictable identifiers. Session cookies are restricted to the host, marked so that scripts cannot read them, and constrained so they are not sent with cross-site requests. A session expires both after a period of inactivity and at an absolute maximum age, and is additionally bound to the browser and network prefix it was created from, so that a stolen session identifier used from elsewhere is rejected. The number of concurrent sessions per user is capped.

State-changing requests require a cross-site request forgery token. Request rates are limited per terminal to contain a device stuck in a retry loop. Response headers restrict how the interface may be embedded, framed, or referenced, and a content security policy authorises the interface’s own scripts by nonce rather than permitting inline script execution generally, which closes the principal avenue for injected script.

Uploaded files are constrained by type and are stored in a location configured so that they cannot be executed by the web server, preventing an uploaded file from becoming a means of running code.

Database Layer Security

Database access is restricted to the application’s own account, which is granted only the privileges it requires. Passwords are stored as hashes and are never recoverable. Device signing secrets are stored encrypted rather than in plain text, so that read access to the database alone does not yield the ability to impersonate a terminal. All queries are parameterised, so that user-supplied values cannot alter the structure of a statement.

Attendance and audit records are treated as append-only history: corrections are recorded as new entries rather than overwriting earlier ones, so that the record of what was originally captured survives any later amendment.

Monitoring Layer Security

Monitoring is implemented through two complementary records. The audit log records ordinary administrative and attendance activity for accountability. The security log records events that carry a security meaning, each classified by severity and carried through a resolution workflow so that an administrator can mark an entry as investigating, resolved, or a false positive rather than merely observing it. The system records thirty-three distinct security event types, grouped in Table 9.

Table 9

Security Event Types Recorded by the System

| Category | Recorded events |
| --- | --- |
| Device and request authentication | DEVICE_UNKNOWN, DEVICE_BLOCKED, DEVICE_SPOOFING, API_KEY_INVALID, API_KEY_EXPIRED, API_KEY_REVOKED_USE, API_KEY_IP_ANOMALY, SIGNATURE_INVALID, TIMESTAMP_EXPIRED, REPLAY_DETECTED, CLAIM_TOKEN_INVALID |
| Network access control | IP_NOT_ALLOWLISTED, BLOCKED_IP_ATTEMPT, RATE_LIMIT_EXCEEDED |
| Account and session security | LOGIN_FAILED, ACCOUNT_LOCKED, SESSION_HIJACK_SUSPECTED, UNAUTHORIZED_ACCESS, PERMISSION_VIOLATION |
| Application attack detection | CSRF_FAILURE, SQL_INJECTION_ATTEMPT, XSS_ATTEMPT, MALICIOUS_UPLOAD |
| Identity and biometric integrity | BIOMETRIC_OVERRIDE_USED, BIOMETRIC_OVERRIDE_REFUSED, FINGERPRINT_FAILURE_THRESHOLD, FINGERPRINT_SLOT_CROSS_DEVICE |
| Attendance and enrolment integrity | UNKNOWN_RFID, DEVICE_CLASSROOM_MISMATCH, SECTION_MISMATCH_REPEATED, GRADE_LEVEL_MISMATCH_BLOCKED, CROSS_DEPARTMENT_ASSIGNMENT_BLOCKED, UNAUTHORIZED_CHANNEL_SUBSCRIBE |

These security mechanisms will be evaluated during the evaluation phase to determine their effectiveness in preventing unauthorized access, detecting system vulnerabilities, and ensuring data confidentiality, integrity, and availability.

Figure 11

Security Architecture Diagram

4.9 Threat Model and Security Considerations

The system adopts a structured STRIDE-based threat model to identify and address common security threats. Table 10 maps each threat category to the specific controls implemented in the system.

Table 10

Threat, Description and Mitigation

| Threat | Description | Mitigation implemented |
| --- | --- | --- |
| Spoofing | Unauthorized impersonation of users or devices | Fingerprint verification for teachers; per-request signature proving possession of the device secret; address-range restriction on terminal traffic |
| Tampering | Unauthorized modification of attendance data | Signature computed over the request content; parameterised queries; append-only attendance and audit history; idempotency keys on state-changing requests |
| Repudiation | Denial of performed actions | Audit log of administrative and attendance activity; separate security log with severity and resolution workflow; recorded biometric override events |
| Information Disclosure | Exposure of confidential data | HTTPS/TLS with an internal certificate authority; role-based access control; device secrets stored encrypted; passwords stored only as bcrypt hashes |
| Denial of Service | Disruption of system availability | Firewall rules and address-range restriction; per-terminal rate limiting; offline capture on the terminal with automatic resynchronisation |
| Elevation of Privilege | Unauthorized privilege escalation | Role-based access control with per-route permission checks; session binding and concurrent session limits; automatic revocation of device credentials showing signs of compromise |

4.10 Security Testing Approach

Penetration testing was conducted in a controlled environment, limited strictly to authorized and owned Local Area Network (LAN) devices, to validate the effectiveness of the implemented security controls. The activities were performed using Wireshark and the Kali Linux toolset.

Secure transport was verified using Wireshark. Traffic between the ESP32 terminals and the server was captured and inspected to confirm that communication is encrypted under HTTPS/TLS and that attendance and authentication data cannot be read in transit.

Resistance to brute-force attacks was tested using Hydra on Kali Linux. Repeated login attempts were directed at the authentication endpoint to confirm that the account lockout and rate-limiting mechanisms respond as designed, blocking access after the configured number of failed attempts rather than allowing unlimited credential guessing.

Network exposure was assessed through port scanning using Nmap on Kali Linux. The server was scanned to enumerate open ports and running services, confirming that only the authorized ports required for system operation are exposed and that no unnecessary services are reachable within the LAN.

In addition, device authentication was tested by submitting forged and unregistered device requests. Each request was rejected by the device-authentication layer and recorded in the system’s security log, demonstrating both real-time rejection and accountability.

All testing procedures were limited strictly to authorized and owned LAN devices to comply with ethical and legal cybersecurity standards.

The results of these tests will be used to classify vulnerabilities based on severity levels using the Common Vulnerability Scoring System (CVSS). Vulnerabilities classified as High Risk (CVSS 7.0 and above) must not be present for the system to be considered acceptable.

Table 11

CVSS Score Interpretation

| CVSS Score | Severity |
| --- | --- |
| 0.1–3.9 | Low |
| 4.0–6.9 | Medium |
| 7.0–8.9 | High |
| 9.0–10.0 | Critical |

In addition to this planned evaluation, transport-layer security was verified during development using an automated certificate test suite, the results of which are presented in Figure 18.

4.11 System Design Considerations

The system design considers efficiency, scalability, reliability, and security. The use of IoT technology enables real-time attendance recording, while the LAN-based setup provides stable communication. The integration of RFID and biometric authentication improves attendance accuracy and prevents unauthorized session initiation. The use of a structured database and web-based interface supports efficient data management and reporting.

The design also supports the collection of measurable data required for evaluation, including attendance timestamps, authentication logs, and system response times. These outputs will be used to validate system performance, accuracy, and efficiency based on the defined experimental metrics.

4.12 System Limitations

The system has several limitations. It depends on the stability of the Local Area Network, and any network interruption may affect real-time data transmission. RFID technology may experience misreads, while biometric authentication may fail to recognise a legitimate user under certain conditions, which is why the controlled password failover described in Section 4.3 exists.

The offline capture mechanism has a defined boundary that should be stated plainly. The terminal holds up to forty taps in the microcontroller’s real-time-clock memory while the server is unreachable, and replays them with their original timestamps once the link returns. That memory survives a reset or a watchdog reboot, but it does not survive a loss of power. If a classroom loses power before connectivity is restored, taps held at that moment are lost.

The system is limited to single-campus deployment and would require further development to support scalability across multiple campuses. The processing power, memory, and storage capacity of the ESP32 may affect performance when handling multiple simultaneous authentication and encryption operations.

Operating an internal certificate authority requires that certificates be renewed and that the root certificate be distributed to terminals when it changes, which adds administrative effort. Device credential rotation likewise adds maintenance as the number of registered terminals grows.

The system was developed as a prototype and will be tested in selected classrooms, which may not fully reflect large-scale deployment scenarios. Security testing will be conducted in controlled environments and may not cover all real-world attack variations.

4.13 Prototype Implementation and System Interface

This section presents the developed prototype as evidence of implementation. The screenshots that follow were captured from the running system, and are grouped by the security function they demonstrate, so that each figure supports a specific claim made earlier in this chapter. The database behind them holds seeded demonstration data, a full school of students, sections, schedules and terminals, so that each screen can be shown populated. The record counts and the number of registered terminals in these figures are therefore illustrative of the system at scale, and are not the two-terminal configuration described in Chapter III for the evaluation.

4.13.1 Hardware Prototype

The classroom terminal integrates the ESP32 microcontroller, the MFRC522 RFID reader and the AS608 optical fingerprint module into a single unit positioned at the classroom entry point. Two terminals were constructed for the prototype, which is what bounds the evaluation to two classrooms as described.

The connections between the microcontroller and the two peripheral modules are presented in Figure 12. The MFRC522 communicates over the SPI bus, while the AS608 uses the second hardware UART at 57600 baud, with its transmit line connected to the pin on which the ESP32 receives.

Figure 12

Terminal Wiring and Connectivity

4.13.2 Authentication and Access Control

Access to the web application begins at a single authenticated entry point. The sign-in screen in Figure 13 is subject to the account lockout and password policy described in Section 4.8, and to the network address restriction described in Section 4.4, which is applied before credentials are examined.

Figure 13

System Sign-In Screen

User accounts are governed by role-based access control. Figure 14 shows the account management interface, in which each account is bound to a role that determines the routes and operations available to it.

Figure 14

User Account and Role Management

Active sessions are visible to the administrator and can be terminated individually or in bulk, as shown in Figure 15. This provides a direct response capability if a session is believed to be compromised, supporting the session controls described in Section 4.8.

Figure 15

Registered IoT Terminal Registry

Every classroom terminal must be registered before it can transmit attendance data. Figure 16 shows the device registry, including each terminal’s identifier, assigned classroom, and connection state.

Figure 16

Terminal Detail and Credential Management

Figure 16 shows the detail view for an individual terminal, from which its credentials are issued, rotated, and revoked, and through which its recent activity and signature failures can be reviewed.

Figure 17

Security Event Log

Figure 17 shows the security event log, in which each recorded event carries its type, severity, source address, and resolution state, implementing the monitoring layer described in Section 4.8 and Table 9.

Figure 18

Audit Trail

Figure 18 shows the audit trail, which records administrative and attendance activity for accountability and provides the non-repudiation control mapped in Table 10.

Figure 19

Fingerprint Enrolment

Figure 19 shows fingerprint enrolment for teaching staff, which establishes the biometric reference used to authorise the opening of an attendance session.

Figure 20

Administrator Dashboard

Figure 20 shows the administrator dashboard, which presents attendance trends, live tap activity, terminal connection status, and a security summary in a single view.

Figure 21

Teacher Portal Dashboard

Figures 21 show the teacher portal, which is restricted by role to the classes a teacher is actually scheduled to teach.

Figure 22

Student Register

Figures 22 show the student register configuration that determine which students are expected in which room at which time, and therefore which taps are valid.

Figure 23

Backup and Restore

Figure 23 shows the backup and restore facility. Backups are encrypted, supporting the availability limb of the CIA triad by allowing the attendance record to be recovered after loss or corruption.

4.13.3 Penetration Testing Results

This section presents the results of the penetration testing described in Section 4.10. The tests were performed from a Kali Linux host on the same Local Area Network as the server, and were limited strictly to the owned and authorized system. Each test corresponds to a threat identified in the security plan (Table 1) and is presented here as evidence that the associated control operates as designed.

Secure transport (HTTPS/TLS). Figure 24 shows the traffic between an ESP32 terminal and the server captured in Wireshark. The TLS handshake is visible, after which all application data is transmitted as encrypted ciphertext. No card identifiers, credentials, or attendance records could be read from the captured stream, confirming that sensitive data is protected in transit.

Brute-force resistance. Figure 25 shows the result of a controlled brute-force login test using Hydra. After the configured threshold of failed attempts, the account lockout and rate-limiting mechanisms blocked further attempts regardless of the password submitted, and no login succeeded. The corresponding lockout event recorded in the Security Center is shown in Figure 26, demonstrating that the attempt was both refused and logged.

Port exposure. Figure 27 shows the Nmap scan of the server. Only the authorized ports required for system operation were found open — the HTTPS port serving the web application and the port serving the real-time monitoring channel — while the database port was not reachable from the LAN. No unnecessary services were exposed.

Device spoofing. Figure 28 shows attempts to access a device endpoint using an unregistered device identifier and a request without a valid signature. Each forged request was rejected by the device-authentication layer with an authentication error, and the matching security events are shown in the Security Center in Figure 29, confirming both real-time rejection and accountability.

Summary. No vulnerabilities classified as High or Critical under the CVSS scale (Table 11) were identified during testing. Every security-relevant action performed during the tests produced a corresponding entry in the audit and security logs, satisfying the log-monitoring criterion of the security plan.

References

Ahmad, S., Qureshi, T., & Jameel, H. (2022). Security challenges in smart educational environments: A case for AES and role-based authentication in IoT systems. International Journal of Advanced Computer Science and Applications, 13(4), 511–519. https://doi.org/10.14569/IJACSA.2022.0130460

Ferrag, M. A., Friha, O., Hamouda, D., Maglaras, L., & Janicke, H. (2022). Edge-IIoTset: A new comprehensive realistic cyber security dataset of IoT and IIoT applications for centralized and federated learning. IEEE Access, 10, 40281–40306. https://doi.org/10.1109/ACCESS.2022.3165805

nternational Organization for Standardization. (2022). ISO/IEC 27001:2022 information security, cybersecurity and privacy protection—Information security management systems—Requirements. https://www.iso.org/standard/27001

Neshenko, N., Bou-Harb, E., Crichigno, J., Kaddoum, G., & Ghani, N. (2021). Demystifying IoT security: An exhaustive survey on IoT vulnerabilities and a first empirical look on internet-scale IoT exploitations. IEEE Communications Surveys & Tutorials, 23(1), 270–313. https://doi.org/10.1109/COMST.2020.3024599

Shrivastava, A., Sharma, P., & Gupta, R. (2023). IoT-based smart attendance monitoring system using RFID technology. International Journal of Innovative Research in Computer Science & Technology, 11(3), 45–50. https://doi.org/10.55524/ijircst.2023.11.3.9

Whitman, M. E., & Mattord, H. J. (2021). Principles of information security (7th ed.). Cengage Learning.